# Data Release Notes

MISSION: LAPS Shimoji 2002

STATUS: Preliminary

DATE: 22 August, 2002

AUTHOR: Maurice Gonella

## 1.	PURPOSE

The purpose of these notes is to provide information on data formatting and payload specifications, and to provide information on the quality of the data set following the manual quality control process.

2.	FILE TYPES & FILE FORMATTING

Up to four different types of data files may be created for each flight, Amdar files (*.amd),  PTU files (*.ptu), a wind files (*.wnd) and metdump excel files (*.xls). File names are of the form mmddaaY.*, where “mm” and “dd” are two digit numbers indicating respectively the month and day that the flight launch took place; “aa” is a two digit number identifying the aircraft that performed the flight; and “Y” is a letter used to differentiate files when an aircraft has made multiple flights on a single day (“A” for first flight of the day, “B” for second flight of the day, etc..).

2.1 WIND FILES: Wind files are comma delimited text files containing wind data for each flight. The first line of each wind file contains header information showing how the data is arranged. Note that time is GPS time (in seconds) and latitude and longitude are given in radians. See Section 3 for a note on converting GPS time. The wind error is calculated based on the least squares fit of wind data. Wind measurements are logged each time the aircraft performs a wind finding manoeuvre. The period between wind finding manoeuvres is variable and is dependant on flight plan structure.

2.2 PTU FILES: PTU files are comma delimited text files containing pressure, temperature, and relative humidity data from each of the two Vaisala RS90 sensors carried on board the Aerosonde. Pressure, temperature and humidity are logged in the wind file approximately every ten seconds during flight. The screenshot below, taken of a sample PTU file shows how the data is arranged. See Section 3 for a note on converting GPS time.

2.3 METDUMP FILES: Metdump files are space delimited text files. Metdump files contain data downloaded from the Aerosonde’s onboard meteorological computer post flight. The screenshot below, taken of a sample metdump file shows how the data is arranged.  Data from the pyrometer (where fitted) is provided as a raw count. Pressure, temperature and humidity is logged approximately every twenty seconds. Wind data is logged each time a wind measurement is made. Between wind measurements, zeros are entered in the wind columns. Under most circumstances the PTU and WND files provide data of higher or equal temporal resolution, however during degraded communication phases of flight (for example the metdump file may contain data not included in the PTU or WND files.

3.	CONVERTING GPS TIME

The PTU and wind files are time stamped with GPS time. GPS time is the time (in seconds) elapsed since 0000 UTC, January 1 1970. To convert to UTC time (in seconds) on the day of flight simply divide the GPS time by 86400 (the number of seconds in a day). The integer gives the number of days since January 1 1970 and can be used to calculate the date of flight. The remainder gives the UTC time of the data point (in seconds) on the day of flight. This process can be performed in excel using the “mod” function. The mod function can be called using an excel equation of the form:

“= mod(A1,86400)”

where A1 is the cell containing GPS time in seconds

4.	PAYLOAD DATASHEETS

4.1 VAISALA RS90 SONDE

### Temperature: F- THERMOCAP® capacitive wire

Measuring range 							+60 °C to -90 °C

Response time (63.2 %, 6m/s flow)

1000 hPa 							0.2 s

10 hPa 								0.5 s

Resolution 								0. 1 °C

### Accuracy

Repeatibility 								0.1 °C

Uncertainty in sounding 						0.5 °C

Reproducibility in sounding

1080 - 100 hPa 							0.2 °C

100 - 20 hPa 							0.2 °C

#### Humidity: H- HUMICAP® thin film capacitor

Measuring range 							0 to 100 % RH

Resolution 								1 % RH

Response time

6 m/s, 1000 hPa, +20 °C 					< 0.5 s

6 m/s, 1000 hPa, -40 °C 					< 20 s

##### Accuracy

Repeatabilityb 								2 % RH

Uncertainty in soundingc 						5% RH

Reproducibility in soundingd  						2 % RH

#### Pressure BAROCAP® silicon sensor

Measuring range 							1080 hPa to 3hPa

Resolution 								0.1 hPa

###### Accuracy

Repeatabilityb

1080 - 100 hPa 							0.4 hPa

100 - 3 hPa 							0.4 hPa

Uncertainty in soundingc

1080 - 100 hPa 							1.5 hPa

100 - 3 hPa 							0.7 hPa

Reproducibility in soundingd

1080 - 100 hPa 							0.5 hPa

100- 3 hPa 							0.3 hPa

4.2 TRIMBLE ACE III GPS BOARD

Performance Specifications

General 			L1 frequency, C/A code (SPS), 8-channel, continuous

tracking receiver, 32 correlators

Update rate 			1Hz

Accuracy

Position			25m CEP (50%) without Selective Availability

Velocity 		0.1 m/sec without Selective Availability (1 sigma)

1 PPS 				±95 nanoseconds (over-determined clock mode)

Dynamics

Acceleration 		4g (39.2 m/sec2)

Motional Jerk 		20 m/sec3

Operational limits 		Altitude <18,000m or velocity <515 m/sec (Either limit may

be exceeded but not both.

Environmental Specifications

Operating temperature 		–40°C to +85°C

Storage temperature 		–55°C to +100°C

Vibration

0.008 g2/Hz 		5Hz to 20Hz

0.05 g2/Hz 		20Hz to 100Hz

3dB/octave 		100Hz to 900Hz

Operating humidity 		5% to 95% R.H. non-condensing, @ +60°C

Altitude 			–400m to +18,000m

PHYSICAL CHARACTERISTIC

S

4.3 AEROSONDE PROPRIETARY WINDFINDING ALGORITHM

In the ‘windfinding-by-maneuver’ method, the aircraft performs a partial circle or S-turn maneuver to calculate winds.  The method uses scalar airspeed from a pitot tube and vector groundspeed from GPS to calculate winds.  Effectively the maneuver recovers vector airspeed information from the scalar airspeed sensor through the maneuver.

RMS Error			Typically < 0.4 m/s

5.	GENERAL DATA ISSUES

- The humidity sensors on the RS90 sondes are not reliable above 95% relative humidity. The sensor will sometimes give relative humidity readings in excess of 100%. Data points between 100% and 120% should be truncated to 100%. Data points in excess of 120% are indicative of a faulty humidity sensor and should be discarded.

- The RS90 sondes require an airflow of at least 6 m/s to operate correctly. Care should be taken when using pressure, temperature or humidity data points acquired while the aircraft is on the ground.

6.	DATA QUALITY REPORTS

- a data quality report will be supplied with the final corrected results.

These are the manufacturers figures and may differ slightly from one installation to another

Standard deviation of differences between two successive repeated calibrations, k = 2

confidence level

2 sigma (95.5%) confidence level (k=2), cumulative uncertainty including: repeatability; long term stability; effects due to measuring conditions; dynamic effect (such as response time); effects due to measurement electronics

standard deviation of differences, in twin soundings divided by 2

These are the manufacturers figures and may differ slightly from one installation to another

Experimentally derived. Cumulative error incorporating errors from algorithm implementation, GPS inaccuracies, and pitot system inaccuracies