Example AMDAR (American Meteorological Data Relay) data:

AMDAR 1600

/// ASONDE001 2021S 11840E 0409 F/// PS287 030 212/015 TB/ S//1 333 F040 VG///=

/// ASONDE001 2021S 11840E 0411 F/// PS285 033 232/013 TB/ S//1 333 F040 VG///=

/// ASONDE001 2022S 11840E 0413 F/// PS287 027 232/012 TB/ S//1 333 F040 VG///=

/// ASONDE001 2022S 11841E 0416 F/// PS292 028 214/014 TB/ S//1 333 F040 VG///=

/// ASONDE001 2021S 11841E 0418 F/// PS286 032 232/013 TB/ S//1 333 F040 VG///=

For weather reports designed to go into the global weather models, there may be a 'destination header' before the 'AMDAR ddhh' line.

Example:

ZCZC PDXSGLCBL

TTAA00 KPDX DDHHMM SDC

The beginning line:

AMDAR dd-hh

is the UTC day of the month and usually the nearest hour for the start of the measurements.

---

Below the text and table describes a line of AMDAR data.

|  | Aircraft ID | Lat & Lon | UTC hhmm | spacer | Temp, tenths of deg C PS=+,MS=- (+19.2 deg C in example) | Humidity % | wind direction and speed (knots) | spacers for unmeasured data | Flight level (hundreds of feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| /// | ASONDE000 | 4548N 12229W | 1553 | F/// | PS192 | 043 | 090/019 | TB/ S//1 333 | F008 | VG/// |

/// is a spacer or start of line code.

Aircraft ID:  ASONDE0xx where xx is the aircraft serial number.

Lat &  Lon in degrees & minutes   ddmmN  dddmmW format

UTC time: hhmm  hours & minutes of the observation.

F///  is a spacer needed by AMDAR indicating data that we are not measuring.

Temperature:  example PS192 is +19.2 deg C, MS192  is -19.2 deg C  (PS=plus/MS=minus)

Humidity:   ex 043  is 43% relative humidity.

Wind:  212/015 is 15 knots from direction 212 degrees.

Altitude:  Flight level in hundreds of feet, ex..  F040 is 40*100 feet or 4000 feet.

VG///  are also spacers.

= is an end of line symbol.